clc;

clear;

close all;

%% Plant model

s = tf('s');

G = 1/(4*s + 1);      % Given system

%% Controller design

% PI controller tuned automatically for the plant

C_pi = pidtune(G,'PI');

- %Kp reduces error quickly

- %Ki removes steady-state error

% If you want to enter your own gains manually, comment the line above

% and use something like:

% Kp = 1.2;

% Ki = 0.35;

% C_pi = pid(Kp,Ki);

disp('PI controller obtained:');

C_pi

%% Closed-loop transfer functions

% Reference to output

T_cl = feedback(C_pi*G,1);

% Disturbance to output (disturbance added at plant input)

% y = G(u + d), u = C(r - y), with r = 0 for regulation

% => Y/D = G / (1 + C*G)

T_d_cl = feedback(G,C_pi);

% Open-loop disturbance response (no controller)

T_d_ol = G;

%% Time settings

t = 0:0.01:30;

% Disturbance: step load change at t = 8 s

d = zeros(size(t));

d(t >= 8) = 1;

%% Simulate disturbance responses

y_ol = lsim(T_d_ol,d,t);      % Open-loop output due to disturbance

y_cl = lsim(T_d_cl,d,t);      % Closed-loop output due to disturbance

%% Plot disturbance response

figure;

plot(t,y_ol,'r--','LineWidth',1.8); hold on;

plot(t,y_cl,'b','LineWidth',2);

xline(8,'k:','Load change at t = 8 s','LineWidth',1.5);

grid on;

xlabel('Time (s)');

ylabel('Frequency deviation');

title('Frequency deviation under sudden load change');

legend('Open-loop','Closed-loop with PI','Location','best');

%% Performance evaluation after disturbance

idx = t >= 8;

t2 = t(idx) - 8;          % reset time axis from disturbance instant

y2 = y_cl(idx);

info = stepinfo(abs(y2),t2);

peak_dev = max(abs(y2));

steady_state_error = abs(y2(end));

fprintf('\nPerformance after disturbance:\n');

fprintf('Peak frequency deviation   = %.4f\n', peak_dev);

fprintf('Settling time              = %.4f s\n', info.SettlingTime);

fprintf('Steady-state error         = %.6f\n', steady_state_error);

%% Stability analysis

disp('Closed-loop poles:');

pole(T_cl)

figure;

pzmap(T_cl);

grid on;

title('Closed-loop pole-zero map');

%% Optional: compare with PID controller

C_pid = pidtune(G,'PID');

T_d_pid = feedback(G,C_pid);

y_pid = lsim(T_d_pid,d,t);

figure;

plot(t,y_cl,'b','LineWidth',2); hold on;

plot(t,y_pid,'m-.','LineWidth',2);

xline(8,'k:','Load change at t = 8 s','LineWidth',1.5);

grid on;

xlabel('Time (s)');

ylabel('Frequency deviation');

title('PI vs PID disturbance rejection');

legend('PI','PID','Location','best');

disp('PID controller obtained:');

C_pid